Determination of Unknown Reactions in a Statically Determinate Beam System

Author: Kathan Patel
Roll No: 23BME067
Course: Engineering Mathematics / Linear Algebra
Tool Used: MATLAB

1. Project Overview

This case study analyzes a statically determinate simply supported beam subjected to external loading.

The objective is to determine the unknown support reactions using:

- Static equilibrium equations

- Matrix formulation

- MATLAB linear algebra solution

The problem demonstrates how engineering mechanics and linear algebra combine to solve structural systems efficiently.

2. Problem Description

A simply supported beam of length L = 6 m is subjected to a point load of 10 kN located 2 m from the left support (A).

Given:

- Beam length (L) = 6 m

- Point Load (P) = 10 kN

- Distance from A (a) = 2 m

Unknowns:

- RA→ Reaction at support A

- RB→ Reaction at support B

Since the structure is statically determinate, the number of unknown reactions equals the number of equilibrium equations.

3. Mathematical Formulation

3.1 Equilibrium Conditions

For a 2D beam system:

∑Fy=0
∑MA=0

3.2 Force Equilibrium

RA+RB-P=0
RA+RB=10

3.3 Moment Equilibrium (About A)

RB⋅L-P⋅a=0
6RB-10(2)=0
6RB=20
RB=3.33 kN

Substituting:

RA=10-3.33=6.67 kN

4. Matrix Formulation

The equilibrium equations can be written in matrix form:

1106RARB=1020

General Linear Algebra Form:

A][R]=[B

Where:

- A→ Coefficient matrix

- R→ Reaction vector

- B→ Load vector

Solution:

[R]=A-1B

5. MATLAB Implementation

Output:

6. Computed Results

| Support | Reaction (kN) |
| --- | --- |
| A | 6.67 |
| B | 3.33 |

Verification:

- RA+RB=10kN

- Moment equilibrium satisfied

7. Engineering Interpretation

- Left support carries higher load because the point load is closer to A.

- The system satisfies all equilibrium conditions.

- Linear algebra provides a systematic way to solve structural equations.

- This approach can be extended to more complex multi-support systems.

8. Stability Concept

For a statically determinate structure:

- Number of unknown reactions = Number of independent equilibrium equations

- Determinant of coefficient matrix ≠ 0

- Unique solution exists

Since:

det(A)=6≠0

The system has a unique stable solution.

9. Learning Outcomes

- Application of equilibrium equations

- Conversion of physical system into matrix form

- Solving simultaneous equations using MATLAB

- Understanding of statically determinate structures

- Verification of engineering results

10. Future Enhancements

- Extend to Uniformly Distributed Load (UDL) case

- Compute Shear Force and Bending Moment Diagrams

- Perform symbolic solution using MATLAB

- Analyze statically indeterminate beam

- Convert implementation to Python (NumPy)

11. References

- Hibbeler – Engineering Mechanics (Statics)

- Bansal – Engineering Mechanics

- Gilbert Strang – Linear Algebra and Its Applications

- MATLAB Documentation – Matrix Computations

Conclusion

This case study successfully determines the unknown reactions of a statically determinate beam using equilibrium equations and matrix algebra.

By solving the system:

A][R]=[B

the support reactions are obtained efficiently using MATLAB.

This demonstrates the powerful application of linear algebra in structural engineering analysis.